Perceptron learning Algorithm for Implementing AND Logic Gate

import numpy as np

# for AND gate (Input/Output)

x_list = np.array([[0,0],[0,1],[1,0],[1,1]])

y_list = np.array([0,0,0,1])

epochs = 4

w,b = np.zeros(2),0

for _ in range(epochs):

for x,y in zip(x_list,y_list):

z = np.dot(x,w) + b

### condition area for y_pred

y_pred = 1 if z >= 0 else 0

E = y - y_pred

print("error : ",E,"<- at epoch",_)

w += E*x

b += E

print(w,b)

Tasks to be performed

- Change the epochs and observe the output

- Change the random weights and bias and observe the convergence of the network.

- Add logic to stop the training when the error is ‘zero’ for all the inputs.

- Repeat the same process for OR gate

- Try the process for XOR gate and show that the network will not stabilize (Converge).